Applications Note

Macintosh OS X Setup Guide For Novatel 3G Merlin PC Card Modems

Table Of Contents

Introduction	3

Requirements	3

Setting Up Your Equipment	3

Set the Network Connection Settings	3

Open an Internet Connection	4

Surf the Internet	4

Disconnect from the Internet	4

Notice: Restricted Proprietary Information

© Copyright Novatel Wireless Technologies Ltd. (2005)

The information contained in this document is the exclusive property of Novatel Wireless Technologies Ltd. All rights reserved.

Unauthorized reproduction of this manual in any form without the expressed written approval of Novatel Wireless Technologies

Ltd is strictly prohibited. This manual may not, in whole or in part, be copied, reproduced, translated, or reduced to any electronic or magnetic storage medium without the written consent of a duly authorized officer of Novatel Wireless Technologies Ltd.

The information contained in this document is subject to change without notice and should not be construed as a commitment by

Novatel Wireless Technologies Ltd. unless such commitment is expressly given in a covering document.

Novatel Wireless Technologies Ltd makes no warranties, either expressed or implied, regarding this document, its merchantability, or its fitness, for any particular purpose.

# Introduction

This guide describes how to connect to the Internet using a Novatel Wireless Merlin U530 or U630

PC Card modem and a Macintosh® PowerBook® with Macintosh OS X

The Novatel Wireless Merlin U530 or U630 enables you to connect to a 3G service provider and the Internet. The modem provides reliable wireless data communications at speeds up to 384 kbps in UMTS and easy to install in any Type II PCMCIA slot of a PowerBook notebook computer.

# Requirements

Before you can connect to a UMTS service provider with a Novatel Wireless Merlin U530 or U630 modem, you will need the following:

- Novatel Wireless Merlin U530 or U630 PC Card modem and an activated UMTS SIM card

- Macintosh PowerBook with Type II PC Card slot and MAC OS X operating system.

- (The following procedure was tested using a Macintosh PowerBook running MAC OS X. It is not guaranteed to work on all Apple computers.)

- UMTS account with a service provider

- Information from your service provider such as user name, password, APN, IP addresses and DNS addresses.

- NVTL_UMTS_Merlin_Modem_Script

# Setting Up Your Equipment

- Power on your Macintosh® PowerBook® computer.

- Verify that the SIM card is inserted properly in the modem

- Plug the modem into your Macintosh PowerBook PC Card (PCMCIA) slot.

- Copy the accompanying modem script NVTL_UMTS_Merlin_Modem_Script to the \\Library\Modem Scripts\ folder.

# Set the Network Connection Settings

In this step, you will set the TCP/IP, PPP, and Modem network connection settings.

- Click the Apple icon and select System Preferences from the menu

- Click the Network icon from the System Preference window.  The Network window appears and the TCP/IP tab is displayed.

- Enter the settings on the TCP/IP tab, as shown in the figure below. Note that some information is optional–check with your service provider to determine what information is required

<FIGURE HERE>

- Select the PPP tab. Enter your Account Name, Password and Telephone Number on the PPP tab, as shown in figure below. These values correspond to the user name, password, and APN supplied to you by your service provider. (Note: Enter the APN in the Telephone Number field)

<Figure>

- Click the PPP Options... button.  Change the settings to match what is shown below.  (Note: You might need to turn off IP header compression on some carrier’s networks.)

<FIGURE>

- Click OK and the view will be reverted back to the Network window

- Select the Modem tab

- Enter your settings to match the settings shown in figure below.  Select NVTL_UMTS_Merlin_Modem_Script for the modem

<Figure>

- Click the Apply Now button.

# Open an Internet Connection

- Click the phone icon located on the MAC menu strip to display the Connect menu

- Click Open Internet Connect.... The Connect dialog box is displayed

- Click the Connect button on the Connect dialog box. The modem attempts to register with the service provider. The status bar message indicates the connection’s status— Status: Contacting PPP Server...

- While a connection is being made, the status bar message is Status: Authenticating User...

- When a connection is made, the status bar message is Status: Connected to and the Connect Time, and IP Address are indicated

- The Disconnect button appears on the window.

# Surf the Internet

Once you have connected to the Internet, you can use your computer and Internet browser software (for example Internet Explorer) to surf the Internet.

To surf the Internet:

- Launch Internet Explorer.

- Select a site or enter the name of the site.

- Visit the site.

# Disconnect from the Internet

To disconnect from the Internet, close your Internet browser and click the Disconnect button on

the Connect dialog box.